FocusGuard

An Intelligent Study Session Monitor

Computer Vision   ·   Data Structures   ·   Statistics

Project Report

Department of Statistics

Eskişehir Technical University

Erasmus+ Programme — South-West University “Neofit Rilski”, Blagoevgrad

Author:  Ramazan Karagöz

Erasmus Coordinator:  Dr. Fatima Sapundzhi

Spring Term, 2026

# Abstract

FocusGuard is a desktop application that helps users monitor their own focus during study sessions. It uses the webcam to track where the user is looking, detects when a phone is visible, and watches which application window is active. From these signals it produces a real-time focus score between 0 and 100, displays it in a live dashboard, and generates a styled HTML report at the end of each session that summarises what happened during the work.

The project was built as an integrated assignment that combines four areas: computer vision (face landmark detection, head pose estimation, object detection), data structures (four custom containers used to manage real-time event streams), statistics (descriptive measures, a normality test, correlation analysis), and software engineering (a multi-threaded desktop application with a graphical interface). The application runs on consumer hardware, stores no images, and works fully offline.

# 1. Introduction

## 1.1 Motivation

Maintaining focus while studying has become increasingly difficult. Smartphones, social media, open browser tabs, and notifications constantly compete for attention, and most students do not have an objective way to measure how much of their study time is actually spent in a focused state. Self-perception is unreliable: people frequently overestimate how productive their last hour was.

FocusGuard addresses this gap by treating the computer itself as the observer. The webcam, which is already pointed at the user, can be used to estimate whether the user is looking at the screen. The operating system already knows which window has focus. A pretrained object detector can recognise a phone in the camera frame. Combining these three signals produces a continuous, objective record of attention without requiring any wearable device or external sensor.

## 1.2 Goals

The project pursued five concrete goals:

- Build a desktop application that produces a focus score in real time using only the webcam and active-window information.

- Use four custom data structures to manage events, statistics, alerts, and time ranges, with each structure chosen for the operation it supports best.

- Generate, at the end of each session, an HTML report that includes descriptive statistics, a normality test, a correlation matrix, and tailored recommendations.

- Distribute the application as a single Windows executable using PyInstaller, so that no Python setup is required for end users.

- Document the system in English (academic submission) and Turkish (end-user installation guide).

## 1.3 Scope

FocusGuard is a personal monitoring tool. It is not designed for proctoring, surveillance, or grading. It only runs while the user explicitly starts a session and stops as soon as the user ends the session. No video frames are stored — only derived numerical events are kept in memory during the session and discarded once the report is generated.

# 2. System Overview

## 2.1 The Three Layers

FocusGuard is organised into three layers, each with a clear responsibility. The detection layer watches the user and the system. The data structures layer remembers what was observed. The analysis layer turns the observations into a final report. The layers communicate through simple Python objects, which makes each one easy to test independently.

| Layer | Responsibility | Main components |
| --- | --- | --- |
| Detection | Watches the user and the operating system in real time. | MediaPipe face mesh, YOLOv8, pygetwindow. |
| Data Structures | Stores events as they arrive and keeps the recent ones quickly accessible. | Deque, hash map, min-heap, interval tree. |
| Analysis | Produces statistics and the HTML session report when the session ends. | pandas, SciPy, matplotlib. |

## 2.2 The Focus Score

The focus score is a single number between 0 and 100 that summarises how focused the user appears to be at any given moment. It is computed from a sliding window covering the last 60 seconds of events. Each event in the window contributes a value between 0 and 100, and the score is the average of these contributions. The contribution table is shown below.

| Event type | Contribution | Meaning |
| --- | --- | --- |
| Focused / productive app | 100 | Ideal: looking at screen, productive window. |
| Other / unknown app | 70 | Neutral: window not classified. |
| Looking away | 55 | Brief loss of attention. |
| Eyes closed | 40 | Sign of drowsiness. |
| Not at desk | 30 | User left the camera frame. |
| Phone visible | 15 | Heavy distraction. |
| Social media | 0 | Worst case. |

This averaging approach has two practical advantages. First, it does not depend on the frame rate: if the camera produces more events per second, the score does not get pushed lower as a side effect. Second, the score recovers naturally as soon as the user returns to focus, because old events fall out of the 60-second window and stop contributing.

# 3. Detection Layer

The detection layer runs three independent detectors on each captured frame. Each detector answers a different question, and they are kept independent so that if one of them fails, the others continue working.

## 3.1 Head Orientation

The head-orientation detector uses the MediaPipe Face Mesh, a Google-released model that produces 468 facial landmarks. A small subset of these landmarks (nose tip, eye corners, mouth corners, chin) is fed into OpenCV's solvePnP function, which estimates the user's head orientation in three dimensions. The orientation is summarised as two angles: yaw (left/right) and pitch (up/down). When either angle exceeds a configured threshold, the user is considered to be looking away from the screen. The thresholds were tuned empirically during development to allow normal small head movements without false alarms.

## 3.2 Eye Closure

The same face mesh provides eye landmarks. The Eye Aspect Ratio, a standard measure first proposed by Soukupová and Čech, is computed from six landmarks per eye: it captures how open the eye is. When the ratio drops below a threshold for several consecutive frames, the eye is reported as closed. Requiring multiple consecutive frames prevents normal blinks from being counted as eye closures.

## 3.3 Phone Detection

Phone detection uses the YOLOv8 nano model from Ultralytics, pretrained on the COCO dataset. Class 67 in COCO is “cell phone.” The detector is called every two seconds rather than every frame, since running an object detection network at the full frame rate would be wasteful and would slow the rest of the pipeline. A phone is reported only when the model's confidence exceeds 45 %.

## 3.4 Active Window Monitoring

The active-window detector uses the pygetwindow library to read the title of the currently focused application window. The title is matched against two keyword lists: a productive list (IDEs, document editors, study tools) and a distraction list (social media, messaging, video platforms). Windows that match neither list are classified as “other.” Keyword matching is intentionally simple — it works well in practice and avoids the privacy and complexity problems that would come with reading window contents.

# 4. Data Structures Layer

The data structures layer is the part of FocusGuard that the data structures course content most directly maps to. Four containers are used, each chosen because it fits a specific operation naturally. The containers are described below in plain language; the goal is to explain why each one was selected, not to discuss formal performance bounds.

## 4.1 Sliding Event Window — Deque

A double-ended queue (deque) stores all events from the last 60 seconds. New events are appended to one end. Old events that fall outside the 60-second window are removed from the other end. A Python list could not do this efficiently because removing items from the front of a list is slow. A deque does both ends equally well, which is why the standard library provides it as a separate type.

## 4.2 Per-Minute Statistics — Hash Map

While the session is running, the application keeps per-minute counts of every event type: how many times the user looked away during minute 3, how many phone detections occurred during minute 7, and so on. These counts are stored in a hash map keyed by minute number. The hash map is exactly the right structure here because the questions asked of it are always of the form “give me the data for a specific minute,” and a hash map answers that kind of question as quickly as a Python dictionary normally does.

## 4.3 Alert Priority — Min-Heap

When several distractions are detected at roughly the same time, the user should not be bombarded with notifications. The application instead picks the most important one and shows that. A min-heap (Python's built-in heapq module) is used to order pending alerts by priority: a phone detection, for example, outranks a brief look-away. The next alert to show is always the one at the top of the heap, and adding a new alert keeps the heap ordered automatically. A separate cooldown mechanism prevents the same alert type from being shown twice in quick succession.

## 4.4 Time-Range Queries — Interval Tree

The interval tree stores every detected event together with the time interval during which it was active. After the session ends, the report uses this tree to answer questions like “which events overlapped the period between minute 12 and minute 15?” An ordinary list of events could answer such questions, but only by scanning every entry. The interval tree skips over irrelevant ranges. The tree was implemented manually rather than using a library, since this was one of the explicit learning goals of the project.

# 5. Analysis Layer and Report

When the user ends a session, the analysis layer reads all the per-minute statistics from the hash map, converts them into a pandas DataFrame, and runs a small set of standard analyses. Each analysis answers a specific question.

## 5.1 Descriptive Statistics

The mean, median, standard deviation, interquartile range, and minimum and maximum focus scores are computed across all minutes. These five numbers give a quick summary of how the session went on average and how variable it was.

## 5.2 Trend Analysis

A simple linear regression is fitted to the per-minute focus scores. The slope of the regression line indicates the overall trend: a strongly negative slope is a typical fatigue curve, while a positive slope suggests a warm-up effect. A nearly flat slope indicates a stable session. The fitted line and the per-minute scores are plotted together in the report so that the trend is also visible.

## 5.3 Normality Test

The Shapiro-Wilk test is applied to the per-minute focus scores to check whether they follow a normal distribution. The test reports a p-value: a value above 0.05 means the data does not significantly deviate from normality, while a value below 0.05 means it does. A non-normal distribution often indicates a session of two parts (for example, a focused first half and a distracted second half), which is useful information by itself.

## 5.4 Correlation Analysis

Pearson correlation coefficients are computed between every pair of distraction types. A high positive correlation between two types means they tend to occur in the same minutes — for example, phone visibility and social media often appear together, since picking up the phone is followed by opening an app. The full correlation matrix is rendered as a heatmap in the report.

## 5.5 Recommendations

Finally, the analysis layer produces a small list of plain-language recommendations based on the most prominent patterns in the session. If more than 15 % of recorded events were social media, the report suggests using a website blocker. If the focus trend declined sharply, it suggests shorter study blocks. The recommendations are deterministic — they come from a fixed set of rules rather than from a language model — which makes them predictable and easy to test.

## 5.6 The HTML Report

The report itself is a self-contained HTML file with embedded charts. The visual design is intentionally editorial rather than dashboard-like: large serif headings, double-rule borders, small caps section labels, and tabular figures throughout. The aim is to make the report feel like a short magazine feature rather than a generic data export. Each session produces a new file, and the file is automatically opened in the user's default browser when the session ends.

# 6. Implementation Notes

Several practical issues came up during development. They are documented here because they shaped the final architecture more than any single design decision did.

## 6.1 Python 3.13 Incompatibility with MediaPipe

Early in development, the application crashed at startup with the error “module mediapipe has no attribute solutions.” The cause was that MediaPipe's recent releases on Python 3.13 do not expose the legacy solutions namespace, while the application code depends on it. The solution was to switch the development and runtime environment to Python 3.11, which has full compatibility. The build script and the installation guide both specify Python 3.11.

## 6.2 Camera Selection on Windows

On Windows machines that have Phone Link enabled, the connected smartphone is registered as the system's default camera (index 0), and the laptop's built-in webcam is moved to a higher index. The application initially opened index 0 by default and tried to capture frames from the phone, which produced a black image. The fix was to add a configurable CAMERA_INDEX setting and to provide a small diagnostic script that scans available indices and reports which one delivers a real image.

## 6.3 Frame-Rate-Dependent Scoring

An earlier version of the focus score accumulated penalties for each negative event. As the frame rate increased, more events were generated per second and the score dropped to zero very quickly, with no way to recover. The score was redefined as a weighted average over the 60-second window, so that frame rate no longer affects the absolute level. The score now moves in a sensible range and recovers as soon as the user returns to a focused state.

## 6.4 UI Flickering

The right-hand statistics panel originally rebuilt all its widgets on every refresh, which caused visible flickering several times per second. The fix was to create the row widgets once at startup and only update their text values during refreshes. This produced a stable, calm interface.

## 6.5 Webcam Black Screen

On some Windows configurations, OpenCV's default camera backend returned an opened capture object that produced only black frames. Using the DirectShow backend explicitly (CAP_DSHOW) solved the problem on every machine where it was tested. The application now tries DirectShow first and falls back to other backends only if necessary.

# 7. Testing

The application was tested informally during development. Each component was exercised in isolation as it was built. The integrated application was then run by the author on his own laptop in real study conditions to verify that the live dashboard behaves as expected and that the report is generated correctly when a session ends.

A formal user study with multiple participants and statistical evaluation of the focus score against ground-truth manual labels was not part of this project. Such an evaluation would be the natural next step and is discussed in the future-work section.

What was verified during development:

- All four data structures behave correctly under unit-style tests with controlled event sequences.

- The focus score remains in the [0, 100] range under all observed conditions, including very high frame rates.

- The HTML report is generated successfully, opens in the default browser, and contains the expected charts.

- The application runs without crashes for sessions of at least one hour on the author's laptop.

- Detection of a phone held up in front of the camera, of looking away from the screen, and of switching to a social-media tab all trigger the corresponding events visibly in the dashboard.

# 8. Limitations

Several limitations are worth recording explicitly:

- The focus score is a behavioural proxy, not a measurement of cognitive engagement. A user can score 100 while staring at the screen without actually thinking about their work, and vice versa.

- The system has not been evaluated against an external ground truth, so the absolute calibration of the score (whether 75 really means “moderately focused”) is based on subjective judgement during development.

- The phone detector recognises a phone in the camera frame but cannot tell whether the user is using it or simply has it nearby.

- Active-window classification relies on keyword matching and is therefore biased toward English and a handful of well-known applications.

- The application does not handle multiple users sharing one webcam — only the largest detected face is used.

- All thresholds were tuned for a specific setup (the author's laptop, ambient lighting, sitting posture). Robustness across very different setups was not measured.

# 9. Conclusion and Future Work

## 9.1 Conclusion

FocusGuard demonstrates that a single integrated project can meaningfully combine computer vision, custom data structures, and statistical analysis into a working desktop application. Each course area contributes a distinct, recognisable component, and the resulting application is genuinely useful as a personal study tool. The development process exposed real engineering issues — frame-rate stability, camera selection, library compatibility — and solving them improved both the application and the author's understanding of the involved disciplines.

## 9.2 Future Work

Several directions would extend the project meaningfully:

- A formal pilot study comparing the focus score against manually labelled video, with multiple participants, would establish how well the score corresponds to actual focus.

- Replacing the keyword-based window classifier with a small machine-learning classifier trained on URL and title patterns would broaden language coverage.

- Integrating ambient signals such as keyboard and mouse activity would refine the score during periods when the camera signal is uncertain.

- Persisting session history across runs would allow longer-term analyses, such as whether the user's typical focus pattern improves over weeks.

- A configurable Pomodoro timer integrated directly into the dashboard would turn the application from a passive monitor into an active study tool.

## 9.3 Closing Statement

Beyond the technical content, the project served its main pedagogical purpose: it required every body of knowledge from the curriculum to come together in a single deliverable. The result is a complete, working, distributable piece of software, which is itself an answer to the question of what an integrated final project should produce.

# References

Lugaresi, C. et al. (2019). MediaPipe: A Framework for Building Perception Pipelines. arXiv:1906.08172.

Jocher, G., Chaurasia, A., Stoken, A., et al. (2023). Ultralytics YOLOv8. https://github.com/ultralytics/ultralytics.

Soukupová, T., & Čech, J. (2016). Real-Time Eye Blink Detection using Facial Landmarks. 21st Computer Vision Winter Workshop.

Bradski, G. (2000). The OpenCV Library. Dr. Dobb's Journal of Software Tools.

McKinney, W. (2010). Data Structures for Statistical Computing in Python. Proceedings of the 9th Python in Science Conference.

Virtanen, P. et al. (2020). SciPy 1.0: Fundamental Algorithms for Scientific Computing in Python. Nature Methods 17(3), 261–272.

Hunter, J. D. (2007). Matplotlib: A 2D Graphics Environment. Computing in Science & Engineering 9(3), 90–95.

Lin, T.-Y. et al. (2014). Microsoft COCO: Common Objects in Context. ECCV 2014.

Shapiro, S. S., & Wilk, M. B. (1965). An Analysis of Variance Test for Normality. Biometrika 52(3-4), 591–611.